## milestone report

| MLA project code: | P.PSH.0338 |
| --- | --- |
| MLA project title: | Sizzle Steaks |
| Project leader: | Fiona Ballantyne |
| MLA project manager/coordinator: | Dean Gutzke |
| Milestone number: | Milestone 3 |

Milestone 3

Report to MLA on full description of

- process,

- raw materials specifications,

- final product specifications and

- equipment and packaging specifications

Please find outlined below complete product specifications

RAW MATERIAL

| Category | Outside Flat |  |
| --- | --- | --- |
| RM Code | 600047 |  |
| Primal Ageing | 5 - 21 Days |  |
| Primal Weight | 3.5 - 5.0kgs |  |
| Storage Temp °C | -2 to 0oC |  |
| Primal Defects | Knife scores Excessive selvage fat Bruising Dark cutters |  |
| NB When supply of Outside Flat is limited, Topside Centre Cut can be used as an alternative Raw Material Code 600113 |  |  |

PROCESSING

| Tempering | 10 hours at-15 to -18oC |  |
| --- | --- | --- |
| Equalization | Overnight (min 8 hours) at -3oC |  |
| Press | Press tempered outside flat to give moulded primal Target temp -2 to -3oC |  |
| Slice | Slice tempered product on Unislicer at 4mm Target temperature -2oC |  |
| Shingle | Shingled automatically on conveyor exiting slicer |  |
| Check Weigh | Correct weight passed for packing Incorrect weight to be corrected manually |  |
| Packing | Packed shingled in pack with fat to bottom |  |
| Pieces Per Pack | 5-9 |  |
| Tray Size | 930b |  |
| Soaker Pad | Yes |  |

KEY QUALITY PARAMETERS

| Slice thickness | 4mm |
| --- | --- |
| Fat levels | Up to 5mm except for seam of fat which can exceed 5mm |
| Steaks per pack | 5-9 |
| Fixed Weight | 385g |

FINAL PRODUCT SPEC

Customer:

Product Name: Beef Sizzle Steak

Product Thickness: 4mm

Fat Thickness: Up to 5mm except at seam which will be more than 5mm

Number of Slices: 5-9

Weight: Fixed Weight of 385g

Tray Type: Black 270 x 170 x 40mm MAP tray

Soaker Pad: Yes

Film: Tray Lidding Film 1050

Carton: Open Top Cardboard Carton

Packs per Carton: 6

Use By Date: 9 Days From Pack

Cost Per Pack: $4.99

EQUIPMENT

Trolleys for tempering

Racks with moveable crates which allow airflow on all sides of the primals to allow the Tempering process to be efficient.

Meat Press

SFK Meat and Bacon Press which gives a uniform size and shape prior to slicing.

This enables the weight to be more controlled and the yield to be maximised as the uniformity gives less waste.

There are 2 moulds for the press. One for smaller primals and one for larger primals.

The 2 moulds prevent the product from being over pressed if the primal is over 5 Kg

Slicing/Portioning

CFS Unislicer which can vary the slice thickness to compensate for the weight of the portion

It cuts the steaks into portions of 5 to 9 steaks at approximately the correct weight

CFS Check 3000 weighing system which check weighs all the portions on a conveyor platform. The check weigher sends information back to the slicer to adjust thickness if required.

CFS Double Decker conveyor system. The check weigher puts the portion on the top or bottom conveyor dependant on the weight.

2). What this product has added in terms of Category Growth to the retail market?

The sizzle steak product has added considerable category growth. Retail sales for the product for the FY09 YTD are over $15 million. This makes the product one of the most successful products launched by B&J, it also obtained new product launch of the year 2008 with Woolworths.

3). An analysis of the improved value for the meat industry of upgrading this cut from a manufactured commodity line to a premium steak line.

Sizzle Steak

Current retails sales 			1,500,000		kg’s (proj F09)

$13.00 per kg						$19 million

This raw material historically would have been sold as trim 50%, corned silversides or exported at trim value.

Retail sales value @ $6.50kg*				$9.75 million

Increased value to beef industry				$9.25 million (approx)

*$6.50 per kg is a weighted average between trim price, corned silversides and export trim pricing that could be expected for the product.

4). Sales of this product will be 1,500 tonnes per annum through one retailer, which will add considerable value to the Australian beef industry.

This year we are on target to manufacture 1,500 tonnes of sizzle steak.

The sizzle steak project has been deemed a great success for all concerned and with the assistance of the MLA we have been able to deliver to the market and the industry positive incremental value.